Install AR2020 Driver

Build DeviceTree:

mkdir AR2020-DTS && cd AR2020-DTS

Place file “ar2020-s2.dts”

dtc -@ -I dts -O dtb -o ar2020.dtbo ar2020-s2.dts

sudo cp ar2020.dtbo /boot/firmware/overlays/

to this folder

Edit  “/overlays/config.txt”

sudo nano /boot/firmware/config.txt

Add these lines:

dtparam=i2c0=on

dtoverlay=ar2020,cam0

dtoverlay=i2c3

dtoverlay=i2c4,pins_6_7,baudrate=400000

Check “I2C”

sudo apt install -y i2c-tools

sudo raspi-config   #Turn on I2C, Interface Option

- Switch I2C channel, switch MIPI channel, and enable the Camera (this step must be repeated every time the Camera loses power).

Add 2 file: “io_script_mipi.py” and “switch_i2c_lane.py”

pip install smbus2

- Turn on  Camera[0] ( [U1], [Sen0], MIPI[0], [I2C Channel 0] ) run 2 file python

- Result Enable: I2C Scan found 0x36:

Install linux Header

sudo apt update && sudo apt install -y git bc build-essential bison flex libssl-dev raspberrypi-kernel-headers

git clone --depth=1 -b rpi-6.12.y https://github.com/raspberrypi/linux.git && cd linux

make bcm2711_defconfig

- Add Driver to Kernel

- Copy “ar2020.c” to “ /home/${USER}/linux/drivers/media/i2c ”

Edit Kconfig:

sudo nano /home/${USER}/linux/drivers/media/i2c/Kconfig

+ Add these lines:

config VIDEO_AR2020

tristate "Onsemi AR2020 sensor support"

depends on I2C && VIDEO_DEV

help

This is a V4L2 driver for the Onsemi AR2020 camera sensor.

Say Y or M to enable support for this sensor.

Edit Makefile:

sudo nano /home/${USER}/linux/drivers/media/i2c/Makefile

+ Add this line:

obj-$(CONFIG_VIDEO_AR2020) += ar2020.o

Build Driver

cd /home/${USER}/linux

make -C /lib/modules/$(uname -r)/build M=$(pwd)/drivers/media/i2c modules CONFIG_VIDEO_AR2020=m

sudo cp drivers/media/i2c/ar2020.ko /lib/modules/$(uname -r)/kernel/drivers/media/i2c/

sudo depmod -a

sudo modprobe -r ar2020

sudo modprobe ar2020

- Done: 🡪 sudo reboot

Capture 1 raw image:

v4l2-ctl --device=/dev/video0 --set-fmt-video=width=5120,height=3840,pixelformat=pBAA

v4l2-ctl --device=/dev/video0 --set-fmt-video=pixelformat=pBAA --stream-mmap --stream-count=1 --stream-to=output.raw

<

- Change parameter:

v4l2-ctl -d /dev/video0 --set-ctrl=test_pattern=0

v4l2-ctl -d /dev/video0 --set-ctrl=exposure=100

v4l2-ctl -d /dev/video0 --set-ctrl=analogue_gain=0

- Decode RAW10 using python:

python raweye.py output.raw -H 3840 -b grbg

python raweye.py output.raw -H 3840 -b grbg -o output.jpeg/.png