USB RNDIS CM4

- CM4:

- Turn on Ethernet Slave:

+  [sudo nano /boot/firmware/config.txt], Add these lines (UART3 for OBC-STM32)

modules-load=dwc2,g_ether

+ At [sudo nano /boot/firmware/cmdline.txt], add this line right after “rootwait”

modules-load=dwc2,g_ether

- Create dhcpcd:

+ sudo apt install dhcpcd

+ sudo nano /etc/dhcpcd.conf    Set static ip USB
		interface usb0

static ip_address=[ip]/nm

+ sudo systemctl enable dhcpcd

+ reboot

- Host connect

- Window: After plugged-in USB, must install driver RNDIS:

Following this link to install: https://www.youtube.com/watch?v=bDZ7P7VzQWE

- Linux: Nothing to do. Set: netmask/gateway (usb slave 192.168.0.7):

Host: Sudo ifconfig usb0 192.168.0.5 netmask 255.255.255.0 up